| What is this document about? Who does it apply to? | The purpose of this Generic site-specific risk assessment is to identify the risks and controls of people working at specific monitoring points. WCO employees, Citizen Scientists, and any site visitors. |
| --- | --- |

WCO CIC Generic Site Specific

Risk Assessment

Assessor		Kelvin Allen

Date			July 2024

Next Review		On Risk Change

Remember to complete a Separate Dynamic Risk Assessment and COSHH assessment for every task undertaken.

Risk Assessment
Likelihood Score (L): 1 – Very Unlikely, 2 – Unlikely, 3- Fairly Likely, 4 – Likely, 5 – Very Likely
Severity Score (S): 1 – No Injury, or Insignificant, 2 – Minor Injury needing, 1st aid, 3 – Medical Attention Needed, 4 – Major Injury Ambulance Needed 5 – Catastrophic Injury or Death
Risk Rating (R) 1-8 Low (acceptable) 9-15 Medium Risk (consider alternatives) 16-25 High (avoid, re-assess or find alternative)

|  | # Hazard and Harmful Effect | # Likelihood # Information | # Severity (S) | # Likelihood (L) | # Risk level (RL) | Control Measures and Method Statements | # Severity (S) | # Likelihood (L) | # Risk level (RL) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Invertebrate Sampling | In water Activities Site Specific Assessment Requirements Immersion / drowning Sudden Immersion / shock Foot entrapment in rocks/mud/weed Ability to access | # Unknown hydrological features like Depth, flow, uneven river bed and bank steepness | 5 | 5 | # 25 | A Formal Site Specific Assessment is Required to assess both Hazards and Risks before any specific tasking. Once completed and Hazards are Defined and understood the appropriate site assessment control measures must be applied as listed below. If the location is assessed as unsafe (R) > 15 then it should be excluded and recorded as such and an alternative site sort, or the Hazard removed and reassessed. | 5 | 3 | 15 |
| Invertebrate Sampling Invertebrate Sampling | Kick Sampling in Water Depths > 1m: . Immersion / drowning Sudden Immersion / shock Foot entrapment in rocks/mud/weed | # Depth, flow, uneven river bed # Silt and mud/marshy ground /weed growth # Unexpected depths and sudden increases in flow upstream. | 5 | 3 | # 15 | No sampling alone (assistant/buddy doesn’t have to be Riverflies trained but preferred). Always wear a lifejacket when in or entering the water Always wear waders when in the water Always have a throw line available Check the water depth and bed stability with a staff or pole. Beware of dynamic water depth and that water levels can rise quickly. Check forecast prior to field work. Never enter a river In flood | 4 | 2 | 8 |
|  | Kick Sampling in Water Depths < 1m: . Immersion / drowning Sudden Immersion / shock Foot entrapment in rocks/mud/weed | # Depth, flow, uneven river bed # Silt and mud/marshy ground /weed growth # Unexpected depths and sudden increases in flow upstream. | 4 | 3 | # 12 | No sampling alone (assistant/buddy doesn’t have to be Riverflies trained but preferred). Always wear a lifejacket when in or entering the water Always wear waders when in the water Check the water depth and bed stability with a staff or pole. Beware of dynamic water depth and that water levels can rise quickly. Check forecast prior to field work. Never enter a river In flood | 3 | 2 | 6 |
|  | Steep and Uneven Bank surface: trips, slips, falls, minor injuries. | # Risk will be affected by weather and surface conditions and awareness of hazard # Physical ability and fitness | 4 | 3 | # 12 | If slippery, do not proceed. If vegetation obscures ability safely enter the water. Location to be Surveyed and assessed prior to commencement of Kick Sample. Volunteers do not attempt to clear it but inform co-Ordinator Wearing of PPE mandatory waders & life vest. Understand your general fitness and abilities, if unsure don’t do it. | 3 | 3 | 6 |
| Water Sampling | Sampling from the roadside or bridge: fall from height, loss of equipment. | # Falling off the bridge or losing equipment from the bridge | 2 | 3 | # 6 | Always wear your Hi Vis vest. Do not lean over the side and keep two feet on the ground. Check for traffic before taking the sample. Never enter the water to retrieve lost equipment. | 2 | 2 | 4 |
|  | Uneven ground surface: trips, slips, falls, minor injuries. | # Risk will be affected by weather and surface conditions. | 2 | 3 | # 6 | Sturdy footwear (walking/wellington boots to be worn). If slippery, do not proceed. If vegetation obscures ability to take samples, Volunteers do not attempt to clear it but inform project officers. | 2 | 1 | 2 |
|  | Water: becoming stuck, hyperthermia, drowning, death. | # Risk will be affected by weather and surface conditions. | 4 | 2 | # 8 | Sampling to be conducted from public access bridge unless site specific guidance suggests bankside sampling. Work away from bank edge. Only sample bankside in dry conditions. | 4 | 1 | 4 |
| Environmental Conditions | Road traffic: being hit by a car, serious injury, death. | # Road traffic: being hit by a car, serious injury, death. | 4 | 2 | # 8 | Wear HI Vis always. Minimise need to walk alongside road. Park in suggested site location. Ensure you can see road in both directions from sample location, check for traffic before taking sample. Consider timing this for outside of peak times and when visibility is poor. | 4 | 1 | 4 |
|  | Sewage: range of infections including hepatitis A. | # More likely near sewage outfalls. | 3 | 2 | # 6 | Cover all cuts before beginning work. Wash hands prior to touching face and before eating. If you develop symptoms, see GP and mention that you work in and around water. | 3 | 1 | 3 |
|  | Cold Weather: hypothermia. Hot Weather: heatstroke, fatigue, dehydration, and sun burn. | # Dependent on time of year task takes place. | 4 | 2 | # 8 | Monitor weather forecast. In cold weather, bring layers and hot drinks. If becoming overly cold/warm, stop and seek shelter. In hot weather take regular breaks for water and shade. Consider adjusting visit time to avoid hottest part of the day. Carry water. | 4 | 1 | 4 |
| Sample Processing | Exposure to Disease & Infection both collecting and undertaken detailed analysis | Risk of cuts and ingesting of unknown chemicals and disease | 3 | 3 | # 9 | Awareness of location risk from sewage outfalls. Provision of sewage maps. Always wear gloves. Wash hands after the survey | 3 | 1 | 3 |
|  | Sample analysis at roadside or bank: breathing problems, injury. | Risk of not being seen by on-coming traffic Overcome by vehicle fumes | 2 | 3 | 6 | Always wear your Hi Vis vest. Park off the road to avoid obstructing traffic and turn off your vehicle. Carry out analysis at a place where you can see the road in both directions. Switch off vehicle once parked with handbrake on. | 2 | 2 | 4 |
| See Separate Dynamic Risk Assessments and COSHH assessments for any chemical being used. |  |  |  |  |  |  |  |  |  |